You can implement ESP32 CAN communication using MCP2515 modules to blink LEDs on transmitter and receiver boards. Connect the modules via CAN_H and CAN_L, and use SPI to interface MCP2515 with ESP32. Below are the hardware connections and example code for both boards.

## Hardware Connections

## MCP2515 to ESP32 wiring

| MCP2515 Pin | ESP32 Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| CS | GPIO5 |
| SO (MISO) | GPIO19 |
| SI (MOSI) | GPIO23 |
| SCK | GPIO18/13 |
| INT | GPIO21/4 |

- Connect CAN_H on transmitter to CAN_H on receiver.

- Connect CAN_L on transmitter to CAN_L on receiver.

- Short a 120 Ω resistor between CAN_H and CAN_L at the ends for proper CAN termination.​

## LED Blinking Circuit

## LED Blinking Circuit

- Connect an LED’s cathode to ESP32 GND.

- Connect anode to a 100 Ω resistor, and then to, e.g., GPIO 2 on ESP32.​

## Example Code

## Transmitter (ESP32 + MCP2515, sends CAN and blinks LED)

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(5);

struct can_frame canMsg;

const int ledPin = 2; // GPIO 2

void setup() {

SPI.begin();

pinMode(ledPin, OUTPUT);

mcp2515.reset();

mcp2515.setBitrate(CAN_125KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

}

void loop() {

digitalWrite(ledPin, HIGH); // LED on

delay(100);

digitalWrite(ledPin, LOW);  // LED off

canMsg.can_id  = 0x100;

canMsg.can_dlc = 1;

canMsg.data[0] = 1; // Send 1 to blink

mcp2515.sendMessage(&canMsg);

delay(900); // repeat every second

}

RECEIVER CODE

#include <SPI.h>

#include <mcp2515.h>

MCP2515 mcp2515(5);

struct can_frame canMsg;

const int ledPin = 2; // GPIO 2

void setup() {

SPI.begin();

pinMode(ledPin, OUTPUT);

mcp2515.reset();

mcp2515.setBitrate(CAN_125KBPS, MCP_8MHZ);

mcp2515.setNormalMode();

}

void loop() {

if (mcp2515.readMessage(&canMsg) == MCP2515::ERROR_OK) {

if (canMsg.can_id == 0x100 && canMsg.data[0] == 1) {

digitalWrite(ledPin, HIGH); // LED blink

delay(100);

digitalWrite(ledPin, LOW);

}

}

delay(10);

}