### Sending SMS in SIM800L GSM Module Working

The SIM800L module supports making calls and sending messages.

Below is the Arduino code for sending a message on the SIM800L. You just need to change the phone number (+XXXXXXXXXXXX)and the text that you need to send, and upload the code.

### SIM800L GSM Module Arduino Code

Here’s the complete SIM800L SMS Arduino code for the same.

#include <SoftwareSerial.h>

// Create a SoftwareSerial object to communicate with the SIM800L module

SoftwareSerial mySerial(3, 2); // SIM800L Tx & Rx connected to Arduino pins #3 & #2

void setup()

{

// Initialize serial communication with Arduino and the Arduino IDE (Serial Monitor)

Serial.begin(9600);

// Initialize serial communication with Arduino and the SIM800L module

mySerial.begin(9600);

Serial.println("Initializing...");

delay(1000);

mySerial.println("AT"); // Handshake test, should return "OK" on success

updateSerial();

mySerial.println("AT+CMGF=1"); // Configuring TEXT mode

updateSerial();

mySerial.println("AT+CMGS=\"+ZZxxxxxxxxxx\""); // Change ZZ with the country code and xxxxxxxxxxx with the phone number to send an SMS to

updateSerial();

mySerial.print("Circuitdigest | circuitdigest.com"); // SMS text content

updateSerial();

mySerial.write(26); // Send the CTRL+Z character to terminate the SMS

}

void loop()

{

}

void updateSerial()

{

delay(500);

while (Serial.available())

{

mySerial.write(Serial.read()); // Forward data from Serial to Software Serial Port

}

while (mySerial.available())

{

Serial.write(mySerial.read()); // Forward data from Software Serial to Serial Port

}

}

If your SIM800L is not sending SMS, check with the SIM card network provider if 2G SMS is available on their network.

### Making Calls on SIM800L Using Arduino

Now, let’s test the call feature. Copy and paste the code given below and change the phone number given in the code (+XXXXXXXXXXXX)  with your recipient's number.

Here’s the complete SIM800L GSM module Arduino code where you need to make the changes.

#include <SoftwareSerial.h>

// Create a SoftwareSerial object to communicate with the SIM800L module

SoftwareSerial mySerial(3, 2); // SIM800L Tx & Rx connected to Arduino pins #3 & #2

void setup()

{

// Initialize serial communication with Arduino and the Arduino IDE (Serial Monitor)

Serial.begin(9600);

// Initialize serial communication with Arduino and the SIM800L module

mySerial.begin(9600);

Serial.println("Initializing...");

delay(1000);

mySerial.println("AT"); // Handshake test, should return "OK" on success

updateSerial();

mySerial.println("ATD+ZZxxxxxxxxxx;"); // Change ZZ with the country code and xxxxxxxxxxx with the phone number to dial

updateSerial();

delay(20000); // Wait for 20 seconds...

mySerial.println("ATH"); // Hang up the call

updateSerial();

}

void loop()

{

}

void updateSerial()

{

delay(500);

while (Serial.available())

{

mySerial.write(Serial.read()); // Forward data from Serial to Software Serial Port

}

while (mySerial.available())

{

Serial.write(mySerial.read()); // Forward data from Software Serial to Serial Port

}

}

Now, upload the code and your recipient should receive a call.

Note:- You can reset the Arduino to make the call again. The same goes for messages

### Technical summary and GitHub Repository

Here’s the GitHub repo where you can find the complete code and the circuit diagram.  We have also provided an overview of a project's key components, implementation details, and functionality, typically covering the hardware/software architecture and design decisions that we have used in this tutorial.